Supplementary Method 2. Description of the particulate respirator and the respirator fitting test.

Disposable particulate respirators (PNTD, Mungyeong, Korea) were used in this study. These particulate respirators are capable of filtering more than 80% of 0.6 µm nonoil particulates, meeting the Korea Food and Drug Administration KF80 standards.5 Before the experiment, a technician provided information on how to wear the particulate respirators according to the guideline provided by the manufacturer. In an open-ended questionnaire format, the technician collected information on specific time and activity that the participants were not able to wear particulate respirators during the experimental period.

A respirator fitting test assessing the face-to-respirator seal was performed for each participant three times in a row using the Mask Fitting Tester MT-03 (SIBATA Science Technology, Saitama, Japan) during the last visit of the entire study. A lower value of a leak test represents better fitting of particulate respirators. The mean of the three measurements was used to estimate plausible personal exposure to particulate matter < 2.5 µm in diameter after particulate respirator use during the experimental period.

The leak test conducted at the end of the study showed low fitting performance of particulate respirators among the elderly participants as higher level indicates more leaking air through particulate respirator. Compared to the first trial of the leak test, the second and third leak tests showed lower mean levels of leaking rates; 73.6%, 71.5%, and 72.8% at the 1st, 2nd, and 3rd tests, respectively. Out of 21 participants, only 14.3% (n = 3), 19% (n = 4) and 29% (n = 6) of participants showed less than 50% of leaking rate or passed each fitting test (Supplementary Table 1).